The Ed Plus CAN Board (905172/906174) is replacing the ED II Board (905173) and the CAN Education Board (905185). Due to the additional features it was necessary to move some of the signals to different pins on the PIC. Following is a summary of the differences:

|  | 905173 | 905185 | 905172 |
| --- | --- | --- | --- |
| RP1 | RA3 | na | Only 2 pots, RP2, RP3 renamed RP1,RP2 |
| E2CS | RA4 | RC1 | na,use internal E2 |
| ROW2 | RA5 | RA3 | RA4 |
| CAN CS | na | RA4 | RC1 |
| ROW1 | RC2 | RA5 | RA5 |
| Encoder A | RB0 | na | RB1 |
| Encoder B | RC0 | na | RC0 |
| Temperature | RC1 | na | RA3 |
| CAN Reset | na | RC0 | RC0 |
| RS 485 DE | na | RC2 | RC2 |

|  | 905173 | 905185 | 905172 |
| --- | --- | --- | --- |
| RA3 | RP1 | Row2 | Temperature |
| RA4 | E2CS | CAN CS | Row2 |
| RA5 | Row2 | Row1 | Row1 |
| RB0 | Encoder A | CAN INT | EncoderA/CAN INT |
| RC0 | Encoder B | CAN RESET | EncoderB/CAN RST |
| RC1 | Temperature | E2CS | CAN CS |
| RC2 | ROW1 | RS485 DE | RS485 DE |

Notes:

1) There is a new jumper (J8) which selects between CAN and Encoder.

2) There are only 2 POTs  on RA0 and RA1

3) There is no longer an external E2 device

4) Digital Temp sensor has been replaced with analog one on RA3

5) If you have code for 905173 the only changes are Row 1, Row2, and Temp